Shire of Dowerin

COUNCIL POLICY – TOWN PLANNING AND BUILDING CONTROL

Policy Number:

Policy Subject:	Outbuildings in Residential Areas

Objectives:		To control the size and height of outbuildings in 					residential areas.

To ensure that the construction of an outbuilding 					does not detract from the general aesthetics of 					the residential area.

Guidelines:		Applications not meeting the above criteria 						and/or of a contentious nature to be referred to 					Council for determination.

Where an application is to be referred to Council 					for consideration, comments from adjoining 						and/or affected owners are to be obtained and 					confirmed in writing.

Policy:

Outbuildings that satisfy the following development criteria may be approved by the Building Surveyor without referral to the Council of the Shire of Dowerin.

Outbuildings that:

- Comply with the Building Code of Australia 1996 (as amended);

- Are constructed of new materials.  Where second hand materials are proposed, the Building Surveyor may require a certification from a practising structural Engineer as to the structural adequacy of the design and/or materials.  The Building Surveyor may also require the cladding of the proposed second hand outbuilding to be painted in an approved colour or renewed.

- Are not attached to a dwelling;

- Are not habitable;

- Are not within the primary street setback area;

- Do not reduce the amount of open space required by the Residential Design Codes to less than the prescribed amount;

- Are setbacks in accordance with the requirements of the Residential Design Codes;

- Are of size, or comprise an aggregate size of outbuildings on one lot, that does not exceed the specifications contained in the table below –

| LOT AREA (m2) | MAXIMUM SINGLE OUTBUILDING (m2) | TOTAL OUTBUILDINGS (m2) | MAXIMUM WALL HEIGHT | MAXIMUM RIDGE HEIGHT |
| --- | --- | --- | --- | --- |
| 500 – 749 | 46 | 62 | 2.4 | 3.6 |
| 750 – 999 | 73 | 97 | 3.0 | 3.6 |
| 1000 – 1249 | 94 | 125 | 3.0 | 3.6 |
| 1250 – 1699 | 117 | 156 | 3.0 | 3.9 |
| 1700 – 2049 | 130 | 202 | 3.0 | 3.9 |
| 2050 – 2999 | 143 | 262 | 3.3 | 4.2 |
| 3000 – 5000 | 157 | 375 | 3.6 | 4.5 |